Department of First year Engineering

Course Code: ESC-102-ELE            Subject: Basic Electrical Engineering           Academic Year – 2025-26

Assignment -IV                                              Semester -I

| Unit –IV AC CIRCUITS |  |
| --- | --- |
| S.NO: | QUESTIONS |
| Que.1 | (a) Define power factor, apparent power, active power and reactive power (b) Draw the power triangle. (c) Z1 and Z2 are in parallel where currents corresponding impedances are I1 = 50∟10 and I2= 20∟300. If the applied voltage is 100∟15V, find true power, reactive power and apparent power in each branch |
| Que.2 | Define the following terms in the context of polyphase AC circuits: i) Symmetric AC supply ii) Phase sequence iii) Balanced Load |
| Que.3 | - Explain the phasor relation for R, L & C elements. - A resistor of 50Ω and inductance of 100mH are connected in series across 200V, 50Hz supply. Determine the following (i) Impedance (ii) current flowing through the circuit (iii) power factor |
| Que.4 | A delta connected balanced load consists of three coils; each of resistance 6 Ω and inductive reactance 8 Ω supplied by 400V, 3-phase AC. Determine i) impedance per phase ii) phase current and line current iii) power factor of the circuit iv) total power absorbed |
| Que.5 | Derive expression for resonance frequency in RLC series circuit and show how reactance’s, impedance and current vary with frequency. |
| Que.6 | Obtain the expressions for instantaneous power and average power in a series R-L circuit excited by purely sinusoidal voltage |
| Que.7 | Define and state the unit of active, reactive and apparent power in case of single phase circuit. |
| Que.8 | Explain power triangle in R-L circuit and estimate three types of powers in a typical motor winding circuit having resistance of 30 ohm and inductive reactance of 40 ohm when connected across 230V AC supply |
| Que.9 | A series RLC circuit of R=40 Ω, L= 50.07mH and a capacitor is connected across a 400V,50Hz, A.C supply. This RLC combination draws a current of 10A.Calculate (i) Power factor of the circuit. (ii) Capacitor value. |
| Que.10 | (a) Define Admittance and impedance (b) The impedances of series circuit are Z1= (6+j8) ohms and Z2 = (8-j6) ohms. If the applied voltage is 120V, find total impedance, current and power factor. Draw the phasor diagram |
| Que.11 | State the relation between: i) Phase voltage and line voltage ii) Phase current and line current in case of balanced delta connected 3-ph load. Using above relations, obtain the expressions for 3-ph active power and 3-ph reactive power |
| Que.12 | Three identical impedances each of 6+j8 ꭥare connected in star across 3-ph, 415 V, 50 Hz ac supply. Determine i) Line voltage ii) Phase voltage iii) Phase current and line current iv) 3-ph active, reactive and apparent power |
| Que.13 | Define impedance. Draw the impedance triangle for R-L & R-C series circuit. |
| Que.14 | Obtain the expression for power in an R-C series circuit when supplied with v = Vm sinwt. |
| Que.15 | A coil of resistance 24ꭥ has a reactance of 32ꭥ, when connected across a single-phase voltage given by v = 566 sin314t. Find: i) Frequency ii) Rms value of current iii) Power factor iv) Equation of the resultant current |
| Que.16 | Determine the average value and rms value of the waveform shown in the fig |
| Que.17 | A sinusoidally varying alternating voltage of 100 V (rms value) with 50 Hz frequency is applied to a circuit find : i) The mathematical equation of the voltage; ii) Time Period iii) The instantaneous voltage when t = 1.667 ms; iv) The time when instantaneous voltage is 100 V; v) Average value of the voltage vi) Maximum value of the voltage. |
| Que.18 | A certain voltage waveform has form factor of 1.15 and peak factor of 1.5. If the maximum value of the voltage is 4500 V, calculate the average and rms values of the voltage. |
| Que.19 | Derive the expression for average power in case of R-C series circuit. Draw voltage, current and power waveforms. |
| Que.20 | Define phase sequence and balanced load. Three phase star connected balanced load with impedance of 9+j12ꭥ per phase is connected across 440V, 50 Hz three phase supply. Calculate phase voltage, phase current, line current, active power and reactive power. |