Department of First year Engineering

Course Code: ESC-102-ELE            Subject: Basic Electrical Engineering           Academic Year – 2025-26

Assignment                                     Semester -I

| Unit –V Introduction to Electric Machines |  |
| --- | --- |
| S.NO: | QUESTIONS |
| Que.1 | What are the two types of 3-phase induction motor? |
| Que.2 | Derive the emf equation of a transformer and hence obtain the voltage and current transformation ratios |
| Que.3 | A transformer is rated at 100 kVA. At full load its copper loss is 1200W and its iron loss is 960W. Calculate: i) the efficiency at full load, UPF ii) the efficiency at half load, 0.8 p.f. iii) the load kVA at which maximum efficiency will occur iv) maximum efficiency at 0.85 p.f. |
| Que.4 | A 500kVA transformer has iron losses of 2600W and copper losses of 7400W at full load. Calculate its efficiency at ¾ full load at unity pf and 0.9 pf lagging. |
| Que.5 | - State any two advantages of three phase transformer over bank of single phase transformers. - Why transformer rating is in KVA? |
| Que.6 | With neat diagram explain the types of three phase induction motor. |
| Que.7 | List the application of various types of DC Motor. |
| Que.8 | Explain briefly the Principle of operation of an induction Motor & the development of Induced Torque in an Induction Motor. |
| Que.9 | A single phase transformer has Z1 = 1.4 + j5.2Ω and Z2 = 0.0117 + j0.465Ω. The input voltage is 6600 V and the turn ratio is 10.6: 1. The secondary feeds a load which draws 300 A at 0.8 power factor lagging. Find the secondary terminal voltage and the KW output. Neglect no-load current. |
| Que.10 | Draw and Explain the working principle of DC machines. |
| Que.11 | The primary winding of a single phase transformer is connected to a 230 V, 50 Hz supply. The secondary winding has 1500 turns. If the maximum value of the core flux is 0.00215 Wb, determine: i) secondary induced emf ii) number of turns in the primary iii) cross sectional area of the core if the maximum value of flux density is 0.1 T iv) Whether it is a step up or a step down transformer? |