Department of First year Engineering

Course Code: ESC-102-ELE            Subject: Basic Electrical Engineering           Academic Year – 2025-26

Question Bank                                      Semester -I

| Unit –V Introduction to Electric Machines |  |
| --- | --- |
| S.NO: | QUESTIONS |
|  | Transformer |
| Que.1 | Define Transformer? Explain working principle of transformer. |
| Que.2 | Calculate primary turns, full load secondary current and maximum flux in core of a single-phase transformer having rating of 10kVA 3300V/220V, 50Hz. EMF per turn is 10V |
| Que.3 | Write formula of copper loss and iron loss in single phase transformer. Full load copper loss and iron loss of a 80KVA, l000V/ 250V, 50Hz, transformer are 1200W and 800W respectively. Calculate efficiency at half load, unity power factor and full load, 0.8 power factor (lagging). |
| Que.4 | Explain the briefly the construction and working of a single phase transformer |
| Que.5 | Write a short notes on regulation and Efficiency of the transformer. |
| Que.6 | The efficiency of a 200 KVA,1-Ф transformer is 98.7% when operating at full-load,0.8 p.f lagging, the iron loss in the transformer is 200 W. Calculate: (i)Full load copper loss (ii) Half load |
| Que.7 | A 200 KVA, 1100/415V, 50Hz single phase transformer has 80 turns of secondary. Calculate the primary number of turns |
| Que.8 | A 100KVA transformer has primary and secondary turns of 400 and 100 respectively. Its primary and secondary resistance and reactance are: R1=0.3Ω, R2=0.015Ω, X1=1.1Ω, X2=0.055Ω, supply voltage is 2400V.Calculate equivalent resistance and reactance on the primary side |
| Que.9 | A 1-Ф,50 HZ transformer has 80 turns on the primary winding and 400 turns on the secondary winding. The net cross sectional area of the core is 200 cm2 .If the primary winding is connected to a 240v,50 HZ supply, determine (i)The emf induced in the secondary winding. |
| Que.10 | Derive the EMF equation of the transformer. |
| Que.11 | What are the losses in the transformer? State the parts in which they take place. |
| Que.12 | The primary winding of a single phase transformer is connected to a 230 V, 50 Hz supply. The secondary winding has 1500 turns. If the maximum value of the core flux is 0.00215 Wb, determine: i) secondary induced emf ii) number of turns in the primary iii) cross sectional area of the core if the maximum value of flux density is 0.1 T iv) Whether it is a step up or a step down transformer? |
| Que.13 | The maximum flux density in core of a 250/1000 V, 50 Hz, 1-ph transformer is 1.2 T. If EMF/turn is 10 V, calculate i) Primary & secondary number of turns ii) area of cross section of core. |
|  | Electrical Motors : |
| Que.14 | With the help of suitable diagrams, explain working principle of three phase induction motor and state any four applications of same |
| Que.15 | Draw sectional view of DC motor and state functions of any three parts |
| Que.16 | State applications of resistive split phase, capacitor start, capacitor start-capacitor run and permanent capacitor type single phase induction motor. |
| Que.17 | Explain the working principle of single-phase Induction motor |
| Que. 18 | Draw circuit diagram for DC shunt and series motor. Write the voltage equations |